| Component | Specification | Quantity |  |
| --- | --- | --- | --- |
| LiPo Battery (11.1V, 5000mAh 3S) | Powers motors + pump | 1 |  |
| LM2596 Buck Converter | 12V → 5V for Pi + STM32 | 2 |  |
| XT60 Connector + wires | Main power distribution | 1 set |  |

| Component | Specification | Quantity |  |
| --- | --- | --- | --- |
| 12V Pump- DC6-12V MINI Aquarium Water Pump R365 | 1–2 L/min flow rate | 1 |  |
| Plastic Reservoir Tank | 3–5 litre, food-grade | 1 |  |
| Silicone Tubing (1m) | 6mm ID, chemical resistant | 1 |  |
| 5V Relay Module (2-channel) | Pump ON/OFF + solenoid valve | 1 |  |

| MPU9250 IMU (9-DOF) | Accel + Gyro + Magnetometer → heading | 1 | 350 |
| --- | --- | --- | --- |
| Neo-6M GPS Module | Budget option for basic positioning | 1 | 500 |

| Logic Level Converter (3.3V↔5V) | Pi ↔ STM32 UART safe comms | 1 |  |
| --- | --- | --- | --- |

| 4WD Robot Chassis Kit | Aluminum frame with 4 motor mounts | 1 | 1,200 |
| --- | --- | --- | --- |
| DC Gear Motors (12V, 150–200 RPM) | High torque for field terrain | 4 | 1,200 |
| Rubber Wheels (65mm) | Anti-slip, wide grip | 4 | 400 |
| BTS7960 H-Bridge Driver | 43A rated, handles 12V motors | 2 | 600 |

X4 encoder disc + ir sensor